WHAT CONSTITUTES AN APPROVED FABRICATOR

PURPOSE

This policy statement is intended to create consistency in how the City of Arvada Building Division will evaluate the competency and the level of quality of proposed fabricators in order to approve the fabricator in accordance with IBC Section 1704.2.5.1.

This policy statement applies to fabrication of all structural load bearing members and assemblies such as: Structural Welding, Moment Resisting Frames, Precast Concrete or Masonry Curtain Walls and Related Products, Masonry, Cold -Formed Steel Framing, Steel Buildings, Wood or Steel Trusses & Joists, Glue Laminated Products, Laminated Veneer Lumber, Parallel Strand Lumber Products, etc.

1702.1 DEFINITIONS

APPROVED FABRICATOR - An established and qualified person, firm or corporation approved by the building official pursuant to Chapter 17 of this code.

INSPECTION OF FABRICATORS

Where fabrication of structural load-bearing members and assemblies is being performed on the premises of a fabricator’s shop, special inspection of the fabricated items shall be required by this section and as required elsewhere in this code.

FABRICATION AND INPLEMENTATION PROCEDURES

The special inspector shall verify that the fabricator maintains detailed fabrication and quality control procedures that provide a basis for inspection control of the workmanship and the fabricator’s ability to conform to approved construction documents and referenced standards. The special inspector shall review the procedures for completeness and adequacy relative to the code requirements for the fabricator’s scope of work.

Exception: Special inspections as required by Section 1704.2.5 shall not be required where the fabricator is approved in accordance with Section 1704.2.5.1.

FABRICATOR APPROVAL

Special inspections during fabrication are not required where the work is done on the premises of a fabricator registered and approved to perform such work without special inspection. Approval shall be based upon review of the fabricator’s written procedural and quality control manuals and periodic auditing of fabrication practices by an approved agency. At completion of fabrication, the approved fabricator shall submit a certificate of compliance to the owner or the owners authorized agent for submittal to the building official as specified in section 1704.5 stating that the work was performed in accordance with the approved construction documents.

WHAT CONSTITUTES AN APPROVED FABRICATOR?

Special inspection is typically required  when  structural  load  bearing  members  and  assemblies  are  fabricated  away from the job site. As stated in the exception to Section 1704.2.5 “Special inspections during fabrication are not required where the fabricator maintains approved detailed fabrication and quality control procedures that provide a basis for control of the workmanship and the fabricator’s ability to conform to approved construction documents and this code.”

AGENCYAPPROVAL

Any fabricator that is currently approved by the one of the following agencies is deemed to be approved by the Arvada Building Division for the applicable discipline:

- AISC for structural steel,

- ICC-IAS Fabricator Inspection Program,

- PCI Plant Certification for precast concrete,

Other equivalent testing agencies approved by the building official.

On-premises special inspection shall be provided for fabricators who are not approved by one or more of the above mentioned agencies. Fabricated items manufactured in a non-approved shop and which are not provided with appropriate on-premises special inspection oversight will be rejected unless on-site special inspection can be done in accordance with Section 1704.

The plans shall identify which items are to be fabricated off-site. The contractor, owner or owner’s representative shall provide documentation to the building official which specifies the name of the fabricator’s shop along with supporting information for evaluation by the building official. Approval by the building official, in writing, is required prior to off-site fabrication of structural load bearing members or assemblies.