Radon mitigation: 8287 w 66th DR.

- Radon Mitigation System: Passive Ventilation System Per Municipality Requirement. Method 1 Preferred. Refer to IRC Appendix F for details.

  - https://codes.iccsafe.org/content/IRC2018/appendix-f-radon-control-methods

- Affected Trades

- Foundation Drains: Install T-fitting and stub-up, provide and spread gravel.

- Flatwork: Supply & install of poly prior to basement pour.

- Insulation: Seal all basement/crawlspace floor penetrations/joints/perimeter expansion joints with SikaFlex Construction Sealant or equivalent.

- Plumbing: Provide and install vent pipe. Provide/Install fan if active system.

- Electrical: Install GFCI in attic for future fan install.